CHAPTER-3

Case 2: One-dimensional steady-state conduction with heat generation

Consider the plane wall in which there is uniform heat generation per unit volume

qWm3, the surfaces are maintained at Ts1 and Ts2 . For constant thermal conductivity, k, the appropriate form of the equation is

d2Tdx2+qk=0     1-D conduction only in x-direction steady-state conduction with uniform heat

generation

d2Tdx2=-qk       ………….   dTdx= -qxk+C1

T(x) =-qx22k+C1x+C2       ……….  Temperature distribution

Hot fluid

T∞1, h1		          Ts2		Uniform heat generation	 inside the plane wall (W/m3)

qgen

Ts1

Cold fluid

T∞2, h2

X=-L         x=0       x=L

Boundary conditions

X=- L        T=Ts1

X=+L        T=Ts2

Substituting BCs will give :

Ts1 =-qL22k-C1L+C2

Ts2 =-qL22k+C1L+C2

Solving the equations simultaneously will give.

T(x) = qL22k1-x2L2+Ts2-Ts12xL+Ts2+Ts12

Heat flux = qx''=-kdTdx=-kqL22k-2x+Ts2-Ts12L=qL2x-(Ts2-Ts12L) k

As you see heat flux is dependent on x under conditions with heat generation

- If you have symmetrical BCs, according to the scheme below:

T0 (max. temperature)

T0

Ts                            Ts

x=-L      x=0      x=+L

x=0      T=T0

x=±L    T=Ts

Temperature distribution will take the following form:

T(x) = qL22k1-x2L2+Ts (1)

Maximum temperature exists at the plane ……………. T0=qL22k+Ts     (2)

Taking the ratio of eqns (1) and (2) will give:

Tx-TsT0-Ts=1-x2L2    ……………Temperature distribution for a system at:

- Steady-state

- Cartesian coordinates

- 1-D conduction (only x-direction)

- Uniform heat generation, q

- Symmetrical BCs