CHAPTER 3

ONE DIMENSIONAL STEADY-STATE CONDUCTION

In this chapter, we treat situations for which heat is transferred by diffusion (conduction) under one dimensional, steady-state conditions.

Case-1 : 1-D , steady-state conduction with no heat generation

Planar wall example (this is the most typical example in cartesian coordinate system)

Heat equation simplifies to ddxkdTdx = 0 under the conditions; steady-state, no heat generation, 1-D conduction

Heat equation tells us that dTdx=C1 = constant

A plane wall that separates two fluids ………….  T= C1 x + C2.  ( T varies linearly with x)

T∞1, h1

hot fluid                    Ts1.                         		cold fluid

TS2

T∞2, h2

qx’’ direction of heat flux

x=0                  x=L

Rt,conv1	           Rt,cond                              Rt,conv2

Thermal circuit

Here we have a  plane wall separating a hot fluid and a cold fluid.

Hot fluid, cold fluid and the planar wall; each possess thermal resistance to heat transfer.

Heot fluid shows Rt,conv1

Cold fluid shows Rt,conv2

Planar wall shows Rt,cond

R denotes resistance and t denotes thermal …………… Rt indicates thermal resistance

Temperature distribution within the planar wall is ………..      T=Ts2-T21L x+Ts1

qx''=-k dTdx=-k Ts2-Ts1L=kTs1-Ts2L    --------- Heat flux

Heat transfer rate ---------     qx=k ATs1-Ts2L  (A is the area normal to the heat transfer)

Thermal resistance

qx=Ts1-Ts2L/k A

Ts1-Ts2 is the driving potential, temperature gradient , ∆T, which leads to the heat transfer in the plane wall

L/k A is the thermal resistance to thermal conduction

So heat transfer rate is …………   Heat transfer rate=Driving potential Thermal resistance

Since qx is constant, we can relate thermal resistances by the equation above.

In terms of overall temperature difference T∞1-T∞2 and the total thermal resistance, Rtot, heat transfer rate