Making balances on multiple units

Doing material balances on multiple processes can be regarded as complex. However, once you have decided on your system, the rest is easy.

System is a part of the process that is surrounded by an imaginary boundary. Some inlets and outlets cross this imaginary boundary (below). System could be:

- Whole process

- Individual units or combination of two or more units

- Any point at which the streams combine or separate

Shortly, the system is the portion of the process that you are interested in.

Choosing system depends on the number of unknowns and given values. You have to start with the smaller number of unknowns and perform DOF analysis for each system in order to progress.

Below example is significant since that is composed of different process units and compounds. Also, that’s very useful to understand the importance of doing DOF analysis because as you check DOF for each system, the easier you proceed.

Recycle streams in processes

Recycling is basically to make use of unreacted or unprocessed compounds by introducing them back to the process. When you come up with a recycle stream in a process, it should not surprise you. You should treat it as a regular stream but have to understand its significance.

Reference: Richard M. Felder, Ronald W. Rousseau, Elementary Principles of Chemical Processes, 3rdEdition.