Chemical Reaction Stoichiometry

Introduction of a chemical reaction into processes requires the consideration of consumption and generation terms in the fundamental material balance equation. Stoichiometry is the study of chemical reactions regarding the moles of reactants consumed and the moles products generated based on the stoichiometric equation(s):

N2 + 3 H2                    2 NH3

Above simply demonstrates the formation of two moles of ammonia from one mole of nitrogen and three moles of hydrogen. The validity of a stoichiometric equation can be checked if the atomic species are balanced or not.

Stoichiometric ratio is the ratio of the moles of any species that participate (either react or generate) in the reaction. This could be  3 moles of H2 reacted1 mole of N2 reacted or 2 moles of NH3 generated3 mole of H2 reacted . This ratio is a useful process specification.

Limiting and Excess Reactants – The reactants that are fed according to the constants in the stoichiometric equation are called limiting reactants. The ones that are fed more than the required according to the stoichiometry are called excess reactants.

Fractional Conversion – In case of any reactant being unreacted, the portion of the reacted consumed should be expressed with a ratio as given below:

Fractional conversion (f) =moles reactedmoles fed

Extent of Reaction – which is a value that has the unit of moles or moles/time, can be used to find out the amounts of the species remaining (after a given time or present in the outlet). Extent of reaction is denoted by the Greek Letter, ξ  Xi and calculated according to the following equation:

ni = ni0+νiξ or ni = ni0+νiξ

ni is the amount of species i remaining (moles or moles/min)

ni0 is the amount of species i initially (moles or moles/min)

νi is the stoichiometric constant of species i

ξ is the extent of the reaction

Once you are given the stoichiometric equation and as well as, ni and ni0 for any species that participate in the reaction, you can calculate ξ. Then you can obtain ni for other species.

Example

Multiple Reactions

During the processes involving processes that require material balances, you may come up with multiple reactions. It should not be challenging if you know how to use the strategy of extent of reaction. This time, you have to designate a different extent of reaction for each reaction such as ξ1, ξ2, ξ3 … and so on.

Assume that you are given the below reactions:

A → B +C	  (Rxn 1)

A + C → 2D        (Rxn 2)

A + B → E +D       (Rxn 3)

If you need to find out nA or nB, you can use the following equation in which the extent of each reaction is included:

nA= nA0-ξ1-ξ2-ξ3 assuming that the stoichiometric constant of A in each reaction is 1.

nB= nB0+ξ1-ξ3 assuming that the stoichiometric constant of B in each reaction is 1.

An important point for the multiple reactions is that the production of desired product will yield some other undesired products. For instance, B is the desired product in the above reactions but D and E are also produced as by-products. So, the objective of the process design should be maximizing the production of B and minimizing the production of D and E. This conclusion brings in some definitions, yield and selectivity.

Yield for B=moles of B formed moles of B that would have been generated in case of no side reactionsand the limiting reactant had completely converted

So, while the nominator represents the real case, the denominator represents the ideal case.

Selectivity B relative to D=moles of B formed moles of D formed

Reference: Richard M. Felder, Ronald W. Rousseau, Elementary Principles of Chemical Processes, 3rdEdition.