Energy and Energy Balances

Since you are an energy engineer, you have to consider then amount of energy that flows in and out of the proceses while performing material balance calculations.

Performing energy balance calculations requires the knowledge of different energy terms such as internal energy, enthaply, kinetic energy and potential energy.

Kinetic energy is the energy due to the  translational motion (velocity) of the system relative to earth.

Potential energy is the energy due to the position of the system relative to some other object.

Internal energy is the energy contained within the system other tha kinetic and potential energy.

A system is any region with a volume and surrounded by boundaries.

The rest of the universe is surroundings.  So, system and surroundings make up the universe.

Isolated system is the one that doesn’t allow material and energy exchange.

Open system is the one that only allows both the material and energy exchange.

Closed system is the one that allows only energy exchange.

Heat exchange happens due to temperature difference between the systems.

Work is the energy flow in or out in response to any driving force such as torque, force or electric voltage – generally regarded as flow work, shaft work and expansion-compression  (P-V work) work.

Energy has units of Joule=N.m, dyne.cm, lbf.ft

State is the point at which system has constant thermodynamic properties such as temperature, volume, and pressure. Position of the system can also be included in these properties.

Intensive property is a property which is independent of the system’s mass such as temperature, pressure, viscosity and density.

Extensive property is the one that is dependent on the mass of the system such as volume, kinetic energy and potential energy.

Specific properties are extensive properties.

An extensive property can be converted into an intensive property by diving the extensive property by the mass (moles) of the system (see below).

A=Am    and    A=An

Both A and A are intensive properties and also called as specific properties.

State and Path Functions

While heat (Q) and work (W) are path functions, internal energy (U), entropy (S) and enthalpy (H)are state functions. So, for U, H and S, their differences (U2-U1 or H2-H1 or S2-S1) are only dependent on the initial and final states of the system, not on the path followed.

Example

Reference: Richard M. Felder, Ronald W. Rousseau, Elementary Principles of Chemical Processes, 3rdEdition.