Phase and equilibrium

Below definition states that the entropy of an isolated system with will tend to increase at constant U and V.

dSU,V ≥ 0

State of the system

S

At equilibrium, the entropy of the system maximizes-------dSU,V = 0

At equilibrium, dSU,V=0

This can also be demonstrated by the plot above. At fixed T and U, entropy of the system will reach a maximum at which the first derivative is 0 and d2SU,V ≤ 0.

At the same time, the following equalities are provided at equilibrium:

dU=0, dA=0, dG=0 but unlike entropy, internal energy, they tend to attain their minimum at eu

Considering isolated systems having multiple phases (phase 1 and 2), the equilibrium criteria can be derived by the following equations. Let’s atart with the derivative form

dS=dUT+ PdVT- μdNT

Since the system is totally isolated, total internal energy, total volume and the total number of moles of for system are all constant (but phases can exchange material and energy):

U=U1+U2 = contant

V=V1+V2  = contant

N=N1+N2  = contant

Their derivatives will give:

0= dU1+dU2				0=dV1+dV2  			0=dN1+dN2

Writing the relation for entropy for an isolated system comprising two different phases at the equilibrium:

dS=dU1T1+dU2T2+ P1dV1T1+P2dV2T2- μ1dN1T1-μ2dN2T2 =0

Since dU1= – dU2, dV1= – dV2, dN1 = – dN2

dS= 1T1-1T2 dU1 +P1T1-P2T2 dV1 –dN1 μ1T1-μ2T2=0

which makes

T1=T2  ………  thermal equilibrium

P1=P2……………….mechanical equilibrium

μ1=μ2……………chemical equilibrium

Degrees of Freedom Analysis for the systems with multiple phases

It is essential to answer this question; how many properties are needed to define the state of a system at equilibrium. Then we have to refer to the following equation:

F = C +2 – P

C represents the number of components, P respresents the number of phases and F is the degrees of freedom. For a system having 2 components and 2 phases at equilibrium, F will be equal to 2. This means that two properties will be enough to define the state of the system such as T and P.

References:

Ismail Tosun, “The Thermodynamics of Phase and Reaction Equilibria”, 2012, Elsevier.

D. Winterbone and A. Turan , "Advanced Thermodynamics for Engineers", 1996, Butterworth-Heinemann.