Thermodynamics of Combustion

Incomplete combustion vs complete combustion

C4H10 + 13/2 O2 = 4CO2 + 5H2O …. 2877 kJ/mol energy released

C4H10 + 9/2 O2 = 4CO + 5H2O ….. 1745 kJ/mol energy released

According to the above equations, 1132 /mol energy lost in the presence of incomplete combustion.

CO is emitted from the combsution of fuels as a result of incomplete combustion resulting either from

-oxygen deficiency

-quenching of combustion processes due to contact with cold surfaces, rapid expansion or a relatively excessive air supply.

How can you prevent CO emissions ?

-Removal from exhaust gases

-Complete combustion – successful oxidation with suitable catalysts

Combustion basically needs flow, mixing and chemical reaction. Chemical reaction is the most essential element.

What types of reactions are combustion reactions?

-Exothermic

-Oxidation

-Fast reactions – the fuels must vaporize for the reaction to occur. They have to be transported into gaseous phase.

Gaseous fuels are more attractive for practical use since they are already in gas phase and go into reaction faster than liquid or solid fuels ------ Gaseous fuels are more mobile, the molecules should collide with each other to go into the reaction.

Factors affeting combustion processes adapted from D. Winterbone and A. Turan , "Advanced Thermodynamics for Engineers", 1996, Butterworth-Heinemann, pg. 208)

| Conditions | Type | Examples |
| --- | --- | --- |
| Time dependence | Steady combustion equipments | Gas turbine, boilers |
|  | Unsteady combustion equipments | Diesel engine |
| Spatial related | 2-D | Bunsen burner |
|  | 3-D | Combustion chamber |
| Fuel-oxidizer mixing | Premixed | Spark ignition engine |
|  | Non- premixed | Diesel engine |
| Phase of the medium | Single | Petrol engines with completely gasified fuel |
|  | Multiphase | Coal fired boilers |
| Flow of reactants | Laminar | Some exceptions |
|  | Turbulent | Boilers |

Enthalpy of reactants and products

HESS LAW-Calculation of heat of combustion of fuels

In order to calculate the heat of combustion, heats of formation of reactants and products from their elements should be considered.

∆Hrxn=∆H°f,products- ∆H°f,reactants

C4H10 + 13/2 O2                      	4CO2 + 5H2O

Formation of CO2 …. C(s) + O2 (g)		   CO2(g)	∆H°f,CO2=-393.5 kJ/mol

Formation of H2O …. H2(g) + 1/2O2 (g)	        H2O (l)  	∆H°f,H2O=-285.8 kJ/mol

Formation of C4H10  …. 4C(s) + 5H2 (g)	        C4H10	∆H°f,C4H10=-125.7 kJ/mol

∆Hrxn= 4 x (-393.5) + 5 x (-285.8) – (–125.7)= –2877.3 kJ/mol butane

References:

Jefferson W. Tester and Michael Modell, “Thermodynamics and Its Applications”, 1996, Prentice Hall.

D. Winterbone and A. Turan , "Advanced Thermodynamics for Engineers", 1996, Butterworth-Heinemann.