Example

An aircraft gas turbine with an isentropic efficiency of 85% receives hot gas from the combustion chamber at 10 bar and 1000°C. It expands this to the atmospheric pressure of 1 bar. If the temperature of the atmosphere is 20°C, determine

(a) the change of availability of the working fluid, and the work done by the turbine if the expansion were isentropic . Then, for the actual turbine, determine

(b) the change of availability and the work done,

Cp=1.100 kJ/kg K, and that the ratio of specific heats, k=1.35.

Example

A steady flow compressor for a gas turbine receives air at 1 bar and 15°C, which it compresses to 7 bar with an efficiency of 83%. Based on surroundings at 5°C, determine

(a) the change of availability and the work for isentropic compression. For the actual process, evaluate

(b) the change of availability and work done, cp=  1.004 kJ/kg K and the ratio of specific heats, k=1.4.

References:

Jefferson W. Tester and Michael Modell, “Thermodynamics and Its Applications”, 1996, Prentice Hall.

D. Winterbone and A. Turan , "Advanced Thermodynamics for Engineers", 1996, Butterworth-Heinemann.